# Homework 02

Justin Adrowski

# Problem 1:

## A) Assume t0 holds the value 0x0000FFF, t1 holds the value 0xEFF01270, t2 holds the value 0x00000001 and t3 holds the value 0x0000001F. What is the value of t2 after the following instructions:

I1, t0 = 0xfffff000

I2, (two’s comp) => fffff000 = -4096, eff01270 = --269027728 => t1 < t0, t1 = 0x00000001

I3, t1 != 0, next line

I4, shift t1 to the right by 31 bits, store in t2 => gets rid of 1 bit, t2 = 0x00000000

I5, exit (no someplace block

## B) Translate C loop into RISC-V with following register maps. All vars = 4byte. How many instructions executed if N = 1, 10, 100, 1000?

addi t2, zero, 0

addi t1, zero, 0

lw s1, 0(s0)

addi s2, x0, 1

for_loop:

bge s2, s3, end_of_for

slli t4, s2, 2

add t1, s0, t4

lw t2, 0(t1)

slt t3, s1, t2

beq t3, zero, max_stay

add s1, x0, t2

max_stay:

addi s2, s2, 1

j for_loop

end_of_for:

N = 1 => 5 Instructions

N = 10 => 77 instructions (86 if max updates every time)

N = 100 => 797 instructions (896 if max updates every time)

N = 1000 => 7997 instructions (8996 if max updates every time)

## C) More RISCV Assembly from C code

li t0, 90

bge a0, t0, SET_A

li t0, 80

bge a0, t0, SET_B

li t0, 70

bge a0, t0, SET_C

li t0, 60

bge a0, t0, SET_D

j SET_F

SET_A: li t1, 'A'

j DONE

SET_B: li t1, 'B'

j DONE

SET_C: li t1, 'C'

j DONE

SET_D: li t1, 'D'

j DONE

SET_F: li t1, 'F'

DONE:

# Problem 2: RISC-V Assembly Language Design

## Bitwise Invert in RISC-V

Xori t1, t2, -1

## Rotate Left immediate (negative immediate implies right rotate)

andi t2, t3,  31 # get shift left amount if sign_ext_imm is negative

li   t4, 32

sub  t4, t4, t2

sll  t0, t1, t2

srl  t1, t1, t4

or   t0, t0, t1

This implementation doesn’t produce any exceptions that I can think of.

## Labels in RISC-V

1 – there’s no room for a full address, let alone three, in a 32-bit instruction.

2 – only the lw and sw instructions can access memory since they only require one memory access port. Having the capability to perform instructions with labels, accessing up to three memory points in one instruction would require a lot more hardware capabilities that would be expensive.

# Problem 3: RISC-V Programming

## C Code implemetaton of strchr

## b) RISC-V function implementation

## RISC-V Test Driver