# Homework 03

Justin Adrowski

# Problem 1)

## Field-by-field

| Or t0, s0, a0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| R-type |  |  |  |  |  |
| Funct7 | Rs2 | Rs1 | Funct3 | rd | opcode |
| 0 | a0 = 10 | s0 = 8 | 6 | t0 = 5 | 51 |
| 0000000 | 01010 | 01000 | 110 | 00101 | 0110011 |

0x00A462B3

1)

| addi t1, zero, 127 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| 127 | 0 | 0 | t1 = 6 | 19 |
| 000001111111 | 00000 | 000 | 00110 | 0010011 |

0x07F00313

2)

3)

| lw t2, 64(t0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| 64 | t0 = 5 | 2 | t2 = 7 | 3 |
| 000001000000 | 00101 | 010 | 00111 | 0000011 |

0x0402A383

| sltu t3, t2, t1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| R-type |  |  |  |  |  |
| Funct7 | Rs2 | Rs1 | Funct3 | rd | opcode |
| 0 | t1 = 6 | t2 = 7 | 3 | t3 = 28 | 51 |
| 0000000 | 00110 | 00111 | 011 | 11100 | 0110011 |

0x0063BE33

4)

| beq t3, zero , loop |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SB-type |  |  |  |  |  |
| Imm(12,10:5) | Rs2 | Rs1 | Funct3 | Imm(4:1,11) | opcode |
| -8 | Zero = 0 | t3 = 28 | 0 | -8 | 99 |
| 1,111111 | 00000 | 11100 | 000 | 1100,1 | 1100011 |

0xFE0E0CE3

5)

| J begin |  |  |
| --- | --- | --- |
| UJ-type |  |  |
| Imm(20, 10:1, 11, 19:12) | rd | opcode |
| -20 | 0 | 111 |
| 11111110110111111111 | 00000 | 1101111 |

0xFEDFF06F

6)

## C) lui sequence

### # two-instruction sequence

| lui t0, 0xFEED20 |  |  |
| --- | --- | --- |
| U-type |  |  |
| Imm(31:12) | rd | opcode |
| FEED20 | t0=5 | 111 |
| 111111101110110100100000 | 00101 | 0110111 |

0xFEED202B7

1)

| addi t0, t0, 0x050 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| 0x050 | 5 | 0 | t0 = 5 | 19 |
| 000001010000 | 00101 | 000 | 00101 | 0010011 |

0x05028293

2)

### # five-instruction sequence

| addi t0, zero, -17 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| -17 | 0 | 0 | t0 = 5 | 19 |
| 111111101111 | 00000 | 000 | 00101 | 0010011 |

0xFEF00293

| slli t0, t0, 12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| 12 | 5 | 1 | t0 = 5 | 19 |
| 000000001100 | 00000 | 001 | 00101 | 0010011 |

0x00C01293

| addi t0, t0, -736 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| -736 | 5 | 0 | t0 = 5 | 19 |
| 110100100000 | 00101 | 000 | 00101 | 0010011 |

0xD2028293

| slli t0, t0, 8 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| 8 | 5 | 1 | t0 = 5 | 19 |
| 000000001000 | 00000 | 001 | 00101 | 0010011 |

0x00801293

| addi t0, t0, 80 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| I-type |  |  |  |  |  |
| Imm | Rs1 | Funct3 | rd | opcode |
| 80 | 5 | 0 | t0 = 5 | 19 |
| 000001010000 | 00101 | 000 | 00101 | 0010011 |

0x05028293

# Problem 2)

## Iterative Implementation

## Recursive Implementation

## Number of instructions

- Iterative

# of instructions

N==0 => 9 instructions

N==1 => 32 instructions (takes first beq branch)

N==2 => 18+16 = 34 instructions (takes first pass of bge branch)

N >= 3 => 18 + 16 + (N-1)*9 instructions (passes over bge N-1 times and executes the next 8 instructions N-1 times, so add 9 onto the 18+16 (N-1) times.

- Recursive
# of instructions

N == 0 => 8 instructions

N = 1 => 32 instructions

N = 2 => 31 instructions

N >= 3 =>     28*F(N) + 29*F(N-1) + 5